The following outlines the Kimberley Ports Authority’s (KPA) Personal Protective Equipment (PPE) requirements for the Port of Broome and the Port of Derby.

These requirements apply to all KPA workers, contractors, port users and visitors accessing wharves, maritime security zones and operational port areas.

Minimum PPE Requirements on the Wharf

The following have been set as the Minimum PPE Requirements for all personnel entering designated operational areas, including wharves at the Port of Broome and Port of Derby:

- Hard hat;

- High visibility clothing;

- Safety footwear (steel cap boots);

- Full length clothing (long shirt and long pants); and

- Safety glasses.

These items must be worn:

In designated areas in accordance with signage and KPA requirements; and

Whenever a risk assessment (e.g. JHA, SWMS, Take-5) identifies applicable hazards

In addition, specific tasks may require the following PPE:

Gloves: for KPA employees, contractors and port users involved in activities and working on the wharf or underdeck, then gloves must be carried at all times.

Personal Flotation Devices (PFD):

  - When working over the red line on the wharf, or where your center of gravity could go over the edge, you must have a fall prevention system in place i.e. handrail or harness. If a fall prevention system is not possible then another control i.e. a PFD must be worn and the risk documented in a procedure / job hazard analysis (JHA);

  - A PFD must be worn when undertaking mooring and unmooring activities, when standing inside of the red hazard line on the wharf where there is no barrier or fall prevention system in place (and the task has been risk assessed) and when accessing a vessel from the landings without a gangway. A PFD is not required when accessing the underdeck landings with all safety rails in place.

Other PPE may be required as per signage, Standard Operating Procedure (SOP) or relevant JHA.

Exceptions to the Minimum PPE Requirements

The following people may be exempt from Minimum PPE Requirements only under controlled conditions:

- KPA workers and contractors who are transferring directly from a vehicle to a designated building (e.g. lunchroom, crib facilities or vessel) These people must not move freely through operational areas;

- Vessel crew or passengers who are transiting directly from a vehicle to a vessel or directly to a designated PPE free zone, (e.g. waiting area at the lunch room / crib facilities, or designated area close to the gangway or berth landing) Note: vessel crew or passengers must not move freely through operational areas  unless they are wearing the Minimum PPE Requirements and displaying their MSIC;

- Small vessel crew undertaking mooring/unmooring duties must wear a PFD in addition to the minimum PPE requirements, however, enclosed shoes are permitted in place of safety boots where appropriate;

- Passengers who are transferring between wharf and vessel or vice versa and are waiting in a designated PPE free area for transport. These passengers must stay within the designated area and be supervised by a port pass holder who is wearing the Minimum PPE Requirements. If passengers are accessing the underdeck they shall meet the PFD requirements detailed above and wear enclosed footwear. It is the responsibility of the charter vessel to ensure passengers accessing the underdeck also wear gloves if required.

  - Note: Passengers are not permitted to move freely through operational areas or cross the red hazard line.  It is up to the Port User (Port Pass holder) to supervise and enforce this at all times; and

- Taxi or bus drivers who remain in the immediate proximity of their vehicle, gangway or designated PPE free area are not required to wear the Minimum PPE Requirements, but must not move freely through operational areas